Research Question:

Does the connectivity of a bike lane network have a greater effect on cycling mode share than total lane coverage, and at what connectivity threshold does cycling adoption become self-sustaining?

- Self-sustaining is where mode stabilizes

Model Architecture:

- Bike lanes:

  - Functionality: improved safety, reduced cost of travel (compared to car), longer travel time

- Grid set up: stylized downtown and residential zones

  - Grid representing city (SingleGrid)

    - Each cell represents a block

    - Bike lanes placed on grid cells (use self.grid.add_property_layer to identify cells with bike lines (Boolean) – reference sugarscape model)

    - Each cell will have a zone attribute (downtown vs residential)

      - Only created at initialization

      - Pre-compute zone cell list

  - Grid represented city:

    - Central cells = downtown (high job density, agents' work nodes cluster here)

    - Outer cells = residential (agents' home nodes cluster here)

  - 200 agents on a 20x20 grid

- Agent specifications:

  - Can choose between two transport modalities: bike and car

  - Each agent is initialized with a baseline probability of choosing to bike (sampled form a uniform distribution)

  - Agent parameters: home, work, mode, social_influence

  - Agents have a fixed commute path; precompute on initialization and store

- Steps:

  - At each step, the agent computes a route cost for biking vs driving

    - Route cost: function of lane coverage on path + observers neighbors choices, updates probability, makes transit (mode) choice

    - cost_bike = time_bike - safety_bonus * lane_coverage_on_path

    - cost_car  = time_car + car_cost

      - This is a fixed cost based on distance (or can do flat penalty homogenous across all agents)

      - Stored for each agent and not re-computed since it’s constant

      - Time_bike = path_length * bike_speed_constant

      - Time_car = path_length * car_speed_constant

    - Social_influence: look at immediate neighbors and compute share of bikers

      - Consider making this a dynamic parameter, but keep R=1 for now for all runs

    - The equations above will include an added term social_influence for observing neighbor agents behavior

    - Update agent probability to bike: p_bike = f(route_cost_difference, social_fraction)

      - P_bike will be modeled as logistic function because:

        - I’m predicting a categorical outcome (bike vs car) based on several predictor variables

      - p_bike = 1 / (1 + exp(-(β * cost_difference + γ * social_fraction)))

        - β: sensitivity to cost

        - γ: social influence

    - Make mode choice using Bernoulli(p_bike)

- Connected network:

  - Connectivity will be measured as largest connected component (LCC) size

    - Lane_coverage = fraction of cells on commute path that have bike lanes

    - Use Von Neumann (4 neighbors, no diagonals) neighborhood to count bike lane cells

    - LCC = largest connected component of lane cells

  - In batch runs, hold total lane-miles constant and vary how edges are distributed

    - High connectivity: cells form a large connected component

      - Place N lanes on cells according to algorithm (e.g., spanning-tree-first)

      - Start from either random cell or near center of grid, and use BFS/DFS until having placed N lanes

    - Low connectivity: same number of edges distributed as isolated segments scattered across the grid

      - Place N lanes on cells randomly from a uniform sample

- Scenarios:

| Scenario | Lane-miles | Connectivity |
| --- | --- | --- |
| Baseline | 0 | 0 |
| Low coverage, fragmented | Low | Low |
| Low coverage, connected | Low | High |
| High coverage, fragmented | High | Low |
| High coverage, connected | High | High |

- Stopping condition:

  - Transport mode share stabilizes

    - Plot equilibrium mode share as a function of connectivity – check for any discontinuity

  - Stabilization is where mode share change < 5% over at least 10 consecutive steps (this is arbitrary right now and I may adjust based on initial model results)

    - Test stopping conditions and comment on the robustness of this threshold

- Additional considerations for analysis/results:

  - The extent to which bike lanes improve safety is an input of the model

    - A parameter that you can tune

  - If bike lanes make you 50% safer, how many bike lanes do you need?

    - The result of the model are the relative parameters

Literature:

https://www.sciencedirect.com/science/article/pii/S2950105924000068

- Provides support for connected bike lanes as a way to encourage active transportation

Walk-bike infrastructure decisions in NYC: https://www.sciencedirect.com/science/article/pii/S0968090X17303194

Simulation of bike patterns: https://www.mdpi.com/2220-9964/10/2/88

https://www.tandfonline.com/doi/full/10.1080/01441647.2024.2337880#abstract

https://www.sciencedirect.com/science/article/abs/pii/S096669231830930X

https://www.urban.org/urban-wire/why-us-cities-are-investing-safer-more-connected-cycling-infrastructure

https://link.springer.com/article/10.1007/s11116-025-10599-5